30 DEGREE DETENT ELBOW

Task Force Tips model # AJ1ST-NT 30 degree elbow shall be provided. For corrosion resistance, the elbow shall be constructed of hardcoat anodized aluminum alloy, have a silver powder coat finish inside and out and include a polymer bearing ring for prevention of galvanic corrosion.

The elbow shall be capable of 360 degree rotation and include a detent device to secure the elbow at one of eight positions to minimize hose kinking and coupling stress.

An eyelet attachment point shall be provided for securing the lanyard or chain for the cap or plug. The elbow shall meet a 900 PSI hydrostatic test and comply with the current NFPA 1964 Standard. The unit shall be covered by a five-year warranty.

The elbow shall be configured with a 5" female NH swivel long handle coupling and a 5" Storz rigid coupling.