HIGH FLOW ELECTRIC REMOTE BALL INTAKE VALVE

Task Force Tips model #AX7NP-NX-RC electric remote control operated lightweight aluminum high flow ball intake valve shall be provided. The unit shall be equipped with an adjustable pressure relief valve under the main valve body with an eight-position adjustable inlet elbow. The valve shall be controlled by a remote panel-mounted push-button switch with LED lights to indicate valve position. The valve can also be controlled with an NFPA compliant slow-close knob which can be configured for left or right hand operation on the valve. A 3/4" bleeder valve shall be provided to exhaust excess air or water from the valve and hose line. A valve-mounted position indicator shall be provided to allow for quick visualization of the status of the valve in the open, closed or partial positions. For maximum corrosion protection the aluminum casting shall be hardcoat anodized, with a powder coat internal and external finish and all components facing the wet side of the valve shall be constructed from hard coat anodized aluminum alloy with corrosion resistant coating.  The electronic controls shall be compatible with 12- or 24-volt DC systems.

The connections shall be: 4" female NH swivel rocker lug with 30 degree swiveling detent elbow and a 6" female NH swivel long handle connection and include polymer bearing strips for prevention of galvanic corrosion. The unit shall have a unique serial number. The valve shall be covered by a 10-year warranty against manufacturing defects and corrosion that affect the valve’s operational performance.