GATED SLOW CLOSE WYE

A Task Force Tips model # AY5NJ-NJ-SC gated slow close wye shall be provided. The lightweight wye shall be configured with a 2-1/2" female NH swivel rocker lug inlet and two (2) 2-1/2" male NH outlets. The outlets shall have quarter-turn aluminum ball valves with full 2-1/2" waterway, and multi-turn knob with valve position indicator. A 1/4" NPT female plugged pressure gauge port, 3/4" NPT female plugged bleeder valve port, and carrying handle shall be standard. For corrosion resistance the wye shall be hardcoat anodized and shall have a durable silver powder coat interior and exterior finish. The unit shall have a unique serial number and be covered by a five-year warranty. The valve shall conform to the current NFPA 1964 standard.