GATED SLOW CLOSE SIAMESE

A Task Force Tips model # AY9ST-NJ-SC gated slow close Siamese shall be provided. The lightweight Siamese shall be configured with a 5” Storz outlet and two (2) 2-1/2" female NH swivel rocker lug inlets. The inlets shall have quarter-turn aluminum ball valves with full 2-1/2" waterway, and multi-turn knob with valve position indicator. A 1/4" NPT female plugged pressure gauge port, 3/4" NPT female plugged bleeder valve port, and carrying handle shall be standard. For corrosion resistance the Siamese shall be hardcoat anodized and shall have a durable silver powder coat interior and exterior finish. The Storz coupling shall be easily configurable as swivel or rigid with a tool. The unit shall have a unique serial number and be covered by a five-year warranty. The valve shall conform to the current NFPA 1964 standard.