LOW PRESSURE AUTOMATIC NOZZLE

Task Force Tips Ultimatic 125, model # BL-BGH-FO automatic nozzle with shutoff shall be provided. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. For corrosion resistance and durability, the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a six (6) position detent flow control stainless steel slide valve, a protective rubber bumper with fog teeth, stainless steel inlet debris screen, laser engraved serial number, and reflective labeling. An integral red pistol grip handle shall be positioned directly below the valve handle. The nozzle shall be covered by a 5-year warranty.

The stream shaper shall have a detent and tactile indicator when in the flashover stream pattern position.

The nozzle shall have a 1" female NH inlet, which allows the nozzle to swivel when tightened, a flow range of 40-400 LPM at 500 KPA (5 BAR) and be designed to accept the Task Force Tips FJ-U low expansion or FJ-UMX FoamJet multi expansion foam attachments. The nozzle shall have a unique serial number and be covered by a five-year warranty.