AUTOMATIC NOZZLE

Task Force Tips Ultimatic 125, model # BTO-125 automatic nozzle tip shall be provided. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a protective rubber bumper with fog teeth, stainless steel inlet debris screen, laser engraved serial number, and reflective labeling.

The nozzle shall have a 1-1/2" female NH rigid inlet with rocker lug coupling, a flow range of 10-125 GPM at 100 PSI and be designed to accept the Task Force Tips FJ-U low expansion or FJ-UMX FoamJet multi expansion foam attachments. The nozzle shall have a unique serial number and be covered by a five-year warranty.