NOZZLE

Task Force Tips model # D1024-MF nozzle shall be provided. The selectable, dual gallonage nozzle shall be furnished in a tip only configuration with flow settings of 10 and 24 GPM at 100 PSI and produce a user adjustable fog or straight stream patterns. The nozzle body shall be constructed from hardcoat anodized aluminum alloy and utilize stainless steel internal components. This tip only nozzle shall have a “twist off” position for break-and-extend operations and incorporate a rubber tip for superior grip and protection. The nozzle shall be furnished with a 1" female Massachusetts Forestry rigid inlet and designed to accept the Task Force Tips FJ-MX-D FoamJet foam aspirating attachment. The unit shall have a unique serial number and be covered by a five-year warranty.