SMOOTH BORE TIP

Task Force Tips model # FSS15075 smooth bore nozzle tip shall be provided. The nozzle shall be constructed from hardcoat anodized aluminum alloy and have a smooth inside taper. The nozzle tip shall be configured with a 1-1/2" female NH inlet and .764" bore. The tip shall be laser engraved with flow and pressure of 150 gpm @ 75 psi and inlet thread size. The unit shall be covered by a five-year warranty.