SELECTABLE GALLONAGE NOZZLE

Task Force Tips Thunder Fog, model # FTS250S selectable gallonage nozzle with shutoff shall be provided. The nozzle design shall allow for straight stream through wide fog patterns with user selectable flows and be able to be flushed without shutting down. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a stainless steel ball valve with quick change polymer valve seat, a protective rubber bumper, stainless steel spinning fog teeth, laser engraved serial number, reflective labeling and five year warranty. The nozzle shall comply with the current NFPA 1964 standard. Nozzle shall be FM approved.

The nozzle shall have five (5) selectable flow settings @ 100 PSI:

95 GPM

125 GPM

150 GPM

200 GPM

250 GPM

The nozzle shall have a 1-1/2" female NH swivel rocker lug inlet, which allows the nozzle to swivel when tightened and be designed to accept the Task Force Tips FJ-MX-FT FoamJet multi expansion foam attachment.