DUAL LOW PRESSURE AUTOMATIC NOZZLE

Task Force Tips Dual-Force, model # HDL-TO low/dual pressure automatic nozzle tip shall be provided. A knob located at the front of the nozzle shall allow for pressure rating adjustments of 75 or 45 PSI. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, a protective rubber bumper with fog teeth, stainless steel inlet debris screen, laser engraved serial number, reflective labeling and five year warranty.

The nozzle shall have a 1-1/2" female NH inlet with rocker lug coupling, a flow range of 95-250 GPM at 75 PSI and be designed to accept the Task Force Tips FJ-H low expansion or FJ-HMX multi expansion FoamJet foam attachments.