PORTABLE ATTACK MONITOR

Task Force Tips Hemisphere model # HPMG161A portable lightweight attack monitor shall be supplied.

The monitor shall have a rotating, 45 degree valved inlet with pressure gauge. The pressure gauge shall read incoming pressure of the hose when the valve is closed. The pressure gauge shall read from 0-300 psi (0-21 bar) and have a protective housing and replaceable lens cover. The monitor shall have a rotating, swiveling outlet that is manually adjustable via a double ball and socket swivel with a manual lock, angled 40 degrees from the main waterway, capable of sweeping 80 degrees and can be rotated about the main waterway a full 360 degrees by a hand crank.

The quarter turn, aluminum half-ball inlet valve shall have a lever type handle and shall lock in the closed position. Monitor outlet shall have a removable stream straightener.

The monitor base shall be capable of attaching to various Task Force Tips Hemisphere Monitor mounts. The monitor’s quick attach mounting base shall have a locking pull pin release to prevent unwanted removal from the mount. The base shall have a separate rotation lock with release lever to allow swiveling the monitor on its mount. The rotation release and mounting release shall lock when monitor is flowing to prevent unwanted movement and disconnection.

An integral carrying handle shall be provided. For corrosion resistance, the monitor shall be constructed from hardcoat anodized lightweight aluminum alloy and have a blue powder coat interior and exterior finish.

The monitor shall be configured with a 75mm Storz swivel inlet and 2-1/2" NH male rigid outlet. The monitor shall be capable of flowing up to 500 GPM. The unit shall have a unique serial number and be covered by a five-year warranty.