NOZZLE SHUTOFF WITH SMOOTH BORE

Task Force Tips model # HVIT24 nozzle with shutoff shall be provided. The shutoff shall be designed with a six (6) position detent (click stop) flow control handle. For corrosion resistance the valve body shall be constructed from hardcoat anodized aluminum extruded alloy. The valve body shall have a retractable outlet thread protector. The size of the orifice and flow and pressure chart shall be laser engraved on the unit.

The nozzle shall be designed to be used with 2-1/2" attack lines. The couplings shall be configured as a 2-1/2" female NH swivel with the ability to continuously swivel when tightened and a 1-1/2" male NH and have a 1-1/8” diameter smooth bore orifice. Unit shall be suitable for use with smooth bore tip, stack tip, or combination fog/straight stream nozzles or used for break-and-extend operations. The unit shall have a unique serial number and be covered by a written five-year warranty.