LOW PRESSURE AUTOMATIC NOZZLE TIP

Task Force Tips Max Matic, model # MD17A low pressure automatic nozzle tip shall be provided. The nozzle design shall allow for straight stream through dense wide fog patterns and be able to be flushed without shutting down. The ribbed rubber bumper shall allow for stream shape adjustment. For corrosion resistance and durability the nozzle shall be constructed from hardcoat anodized aluminum alloy, a protective rubber bumper with fog teeth, laser engraved serial number, reflective labeling and five year warranty.

The nozzle shall be suitable for foam solution application and designed to accept the Task Force Tips FJ-MX-MD multi-expansion air aspirating attachment. The nozzle shall have a 2-1/2" female NH rigid rocker lug coupling and a flow range of 100-500 GPM at 80 PSI.