FIXED GALLONAGE NOZZLE

Task Force Tips Metro 1, model # ME1V-212 fixed gallonage nozzle shall be provided. The nozzle design shall allow for straight stream through wide fog patterns with user selectable flows and pressures and be able to be flushed without shutting down. For corrosion resistance and durability, the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a stainless steel slide valve with detent flow control for turbulence free flow, a rubber bumper with "power fog" teeth, stainless steel inlet debris screen and reflective labeling. The nozzle shall conform to the current NFPA 1964 standard.

The nozzle shall have the capability to change flow/pressure discs, available separately. The nozzle shall be configured with 150 GPM at 50 PSI (K factor 21.2) disc.

The nozzle shall have a 1-1/2" female NH swivel rocker lug inlet that allows the nozzle to swivel when tightened and be designed to accept the Task Force Tips FJ-LX-HM low expansion or FJ-MX-HM medium expansion foam attachments. The unit shall be covered by a five-year warranty.