FIXED GALLONAGE NOZZLE

Task Force Tips Metro 2, model # ME22VPGI-262 fixed gallonage nozzle shall be provided. The nozzle design shall allow for straight stream through wide fog patterns with user selectable flows and pressures and be able to be flushed without shutting down. For corrosion resistance and durability, the nozzle shall be constructed from hardcoat anodized aluminum alloy, have a nylon ball valve with detent flow control, a rubber bumper with "power fog" teeth, stainless steel inlet debris screen and reflective labeling. An integral pistol grip handle shall be positioned directly below the valve handle. The nozzle shall conform to the current NFPA 1964 standard.

The nozzle shall have the capability to change flow/pressure discs, available separately. The nozzle shall be configured with 185 GPM at 50 PSI (K factor 26.2) disc.

The nozzle shall have a 2-1/2" female NH swivel rocker lug inlet that allows the nozzle to swivel when tightened and be designed to accept the Task Force Tips FJ-H low expansion or FJ-HMX medium expansion foam attachments. The nozzle shall have a unique serial number and be covered by a 5-year warranty.