SELECTABLE GALLONAGE NOZZLE

Task Force Tips model # QF150S-GFR QuadraFog selectable gallonage nozzle with shutoff shall be supplied. The nozzle shall be capable of producing a variety of stream patterns from a straight stream to a narrow fog or extremely wide fog. The nozzle shall have clear anodized aluminum spinning fog teeth. For corrosion resistance the nozzle shall be constructed from hardcoat anodized aluminum with stainless steel ball valve that has a quick change polymer seat. An integral pistol grip handle shall be positioned directly below the valve handle. The nozzle shall be configured for flow settings of 20-40-100-150 l/min at 6 bar, have a GFR20 female inlet and have the ability to flush without shutting down. The nozzle shall conform to the current NFPA 1964 standard.

The nozzle shall be designed to accept the TFT FoamJet, FJ-DQ low expansion or FJ-MX-DQ multi-expansion aspirating attachments. The nozzle shall have reflective labeling, protective front bumper, laser engraved serial number and a five year warranty.