SAFE-TAK PORTABLE MONITOR BASE

Task Force Tips Safe-Tak 1250, model # XFH-1NN portable monitor base shall be provided. The monitor shall include a Safe-Tak, spring loaded butterfly valve designed to rapidly reduce the water flow by 90 percent in the event that contact with the ground is lost. The device shall include an integral carrying handle, four folding stainless steel legs with replaceable tungsten carbide spikes and an anchoring strap (attached to a protective cap) designed to be stored inside the waterway. The butterfly valve shall have a reset handle located near the inlet to allow the water flow to be reestablished once the base is properly stabilized. The base shall allow an optional pressure relief valve to be installed.

The base shall be constructed from hardcoat anodized aluminum and have a red powder coat interior and exterior finish. The inlet shall be configured with one (1) 3-1/2" female NH swivel rocker lug coupling. The outlet shall allow quick attachment of a Crossfire monitor top. The unit shall have a unique serial numbers and be covered by a five-year warranty.