1250 GPM FIXED LOCATION MONITOR

Task Force Tips model # XFIH-DL1A-01 firefighting monitor shall be supplied. The monitor shall be constructed from hardcoat anodized aluminum with a rally red powder coat internal and external finish. The monitor shall have the ability to horizontally rotate 360 degrees and have a vertical elevation travel of 135 degrees from 90 degrees above to 45 degrees below horizontal. Rotation and elevation shall each be controlled by a handwheel with stainless steel worm gear. Grease fittings shall be located on all pivot points for durability and ease of maintenance and a threaded port shall be provided for a pressure gauge. The monitor shall have an automatic drain valve.

The monitor shall be configured with a Task Force Tips code RLF female threaded inlet to mount directly on Task Force Tips Extend-A-Gun RC3 telescoping deck pipe, and 2-1/2" male NH outlet threads, be capable of flowing 1250 GPM and have a maximum operating pressure of 175 PSI. The unit shall have a unique serial numbers and be covered by a five-year warranty.