PORTABLE DECK GUN MONITOR TOP

Task Force Tips Crossfire, model # XFT-BJ portable monitor shall be provided. This top only portion with quick release swivel joint shall be designed for use on truck mounted risers and TFT Safe-Tak or Stow-A-Way series portable bases. The monitor shall include safety devices that include a locking button which locks the quick release lever when monitor is pressurized, and a 1/4 turn rotational lever lock that secures the horizontal rotation and provides a visual indication that the monitor rotation is locked. For corrosion resistance the monitor shall be constructed from hardcoat anodized aluminum with a rally red powder coat interior and exterior finish.

The monitor shall have a 3-1/4" waterway for delivery of up to 1250 GPM with low friction loss. Vertical elevation shall be controlled through use of a handwheel controlled stainless steel worm gear which allows full travel to the safety stop point of 35 degrees above horizontal with seven rotations of the wheel. When positioned on a truck mounted riser the monitor shall be able to be used below the 35 degree stop point through release of the spring loaded safety pin.

An automatic drain to remove remaining water to avoid freezing shall be included. Integral stream straightening vane and pressure gauge shall be included. The monitor shall be configured with a Crossfire inlet connection and 2-1/2" male BSP outlet. The unit shall have a unique serial number and be covered by a five-year warranty.