PORTABLE ATTACK MONITOR PACKAGE

Task Force Tips Blitzfire model # XXC-42-NBNH1 portable lightweight attack monitor package comprising of monitor and stack tip nozzles shall be supplied.

The monitor model number XX111A shall include an integral safety shut off valve that will automatically shut down the water flow in the event control of the monitor is lost. The water flow is controlled by a six-position detent, turbulence free slide valve. The detent flow control shall also function as the reset for the safety shut off valve to resume water flow.

The water inlet shall pivot up and down to allow for stability on uneven surfaces. The discharge has vertical elevation travel between 10 and 40 degrees above horizontal with a ball and socket joint. The same ball and socket joint shall allow for horizontal adjustment of 20 degrees left and right of center. The monitor shall have a flow rating from 100 to 500 GPM (400-2000 l/min). Outlet has removable stream shaper. The monitor shall have a 2-1/2” (64 mm) female NH swivel rocker lug inlet and a 2-1/2” (64 mm) male NH outlet.

The monitor shall have folding stabilization legs with carbide tips, which are concealed in the carry and stow positions. An integral carrying handle that also functions as an attachment point for the included anchor strap shall be provided. For corrosion resistance, the monitor shall be constructed from hardcoat anodized aluminum alloy and have a blue powder coat interior and exterior finish.

Task Force Tips model # MST-3NJ smooth bore stacked tip set shall be provided. For corrosion resistance the tip set shall be constructed from hardcoat anodized aluminum alloy. The set shall consist of three (3) tips with the base tip having a 2-1/2" (64 mm) female NH swivel inlet and 1.5” (38 mm) orifice outlet. The other tip sizes shall be 1.25” (32 mm) and 1” (25 mm). The two larger tips shall have 1-1/2" (38 mm) male NH threads. Each tip shall be laser engraved with a flow/pressure chart and orifice size.

The monitor shall have a unique serial number and all components shall be covered by a five-year warranty.