PORTABLE ATTACK MONITOR

Task Force Tips Blitzforce model XXL111A portable lightweight attack monitor shall be supplied.

The water flow is controlled by a positive locking lever controlling a ball valve. The 2-1/2” (64 mm) water discharge has vertical elevation travel between 20 and 60 degrees above horizontal with a ball and socket joint. The same ball and socket joint shall allow for horizontal adjustment of 20 degrees left and right of center. The monitor shall have a flow rating from 100 to 500 GPM (400-2000 l/min). Outlet has removable stream shaper.

The monitor shall have folding stabilization legs with carbide tips, which are concealed in the carry and stow position. An integral carrying handle and anchor strap shall be included. For corrosion resistance, the monitor shall be constructed from hardcoat anodized aluminum alloy and have a silver powder coat interior and exterior finish. The monitor shall have a 2-1/2” (64 mm) NH female swivel rocker lug inlet and a 2-1/2” (64 mm) male NH outlet.

The monitor shall have a unique serial number and shall be covered by a five-year warranty.