500 GPM TILLER BAR MONITOR

Task Force Tips Tornado model # Y2-T84A tiller bar controlled monitor shall be provided. The monitor shall operate with a tiller bar for rotation and elevation adjustment.

The monitor shall have the following capabilities: full horizontal rotation with 360 degree travel, with field changeable rotation stops at 180 degrees, 135 degrees of vertical travel with field changeable elevation travel stops at 45 degrees above and 20 degrees below horizontal, flow capability of up to 500 GPM with no more than 19 PSI loss, maximum operating pressure of 200 PSI. Locking knobs for horizontal and elevation travel shall be provided.

For resistance to corrosion the monitor shall be constructed from hardcoat anodized aluminum with a silver powder coat interior and exterior finish.

The monitor shall be configured with a 2" female NPT inlet adapter to 2-1/2” NH quick disconnect with locking pin and 1-1/2" male NH outlet. The unit shall have a unique serial number and be covered by a five-year warranty.