1500 GPM ARTICULATING MONITOR

Task Force Tips model # Y35-D12A Radius articulating monitor shall be supplied. The monitor shall be constructed from hardcoat anodized aluminum alloy with a silver powder coat internal and external finish. The monitor shall have the ability to horizontally rotate 360 degrees and have a vertical elevation travel of 135 degrees from 90 degrees above to 45 degrees below horizontal.

The outlet section of the monitor can be manually articulated up, raising the nozzle outlet position to 22.9” above the inlet flange. A spring-loaded locking pin with knob shall lock the monitor in the raised or lowered position for flowing. A large knob on the outlet elbow for ease of articulation operation shall be standard. A “deck monitor raised” light, position sensor, relay, and bracket kit for 12-volt DC systems shall be included.

Rotation and elevation shall each be controlled by a crank with stainless steel worm gear. Grease fittings shall be located on all pivot points for durability and ease of maintenance and a threaded port shall be provided for a pressure gauge. The monitor shall have an automatic drain valve.

The monitor shall be configured with a 4” ANSI 150 flange inlet with 4” female NPT threads and 3-1/2" male BSP outlet threads, be capable of flowing 1500 GPM and have a maximum operating pressure of 200 PSI. The unit shall have a unique serial number and be covered by a five-year warranty.