1250 GPM MANUAL AERIAL PLATFORM MONITOR

Task Force Tips Monsoon model # Y4-M25A-P manually controlled aerial platform monitor shall be provided. The monitor shall operate with a handwheel for elevation adjustment and crank handle for rotation adjustment, which shall control stainless steel worm gears.

The monitor shall have the following capabilities:

Horizontal rotation with travel 90 degrees left and right of center;

90 degrees of vertical travel with stops at 45 degrees above horizontal and 45 degrees below horizontal;

Field changeable rotation stops at 45 and 90 degrees left and/or right of center and 30degrees above and/or 30 degrees below horizontal;

Flow capability of 1250 GPM with friction loss no more than 6 PSI;

Maximum operating pressure of 200 PSI

For resistance to corrosion the monitor shall be constructed from hardcoat anodized aluminum with a silver powder coat interior and exterior finish. A threaded port for an optional pressure gauge shall be provided. The unit shall have a unique serial number and be covered by a five-year warranty. Monitor shall be FM approved.

The monitor shall be designed with a unique waterway that minimizes the path of travel, reduces friction loss and turbulence, and produces a far reaching water stream. The monitor shall be configured with a 4" ANSI 150 flange inlet and 2-1/2" male NH outlet.