REMOTE MONITOR VALVE KIT

Task Force Tips model # YE-VKG-PL electric gateable valve kit for Task Force Tips Tornado, Hurricane, and Monsoon 1250 series remote control monitors shall be installed. The valve package shall include a stainless steel valve with controller mounted on valve, and an interface box for connection of joystick or other controller, monitor and power. The valve kit electrical components shall be compatible with 12 or 24-volt direct current.

The enclosure shall be weatherproof and utilize weatherproof components such as a silicone seal, hardware with o-rings and liquid tight electrical connections with strain relief fittings.

The enclosure shall include the following:

A preconnected 20 foot, four-conductor cable with waterproof, four-pin connection plug for power and communications connection to the monitor;

A preconnected 20 foot, six-conductor cable with waterproof, six-pin connection plug for connection to valve controller;

A preconnected 15 foot, two conductor cable with stripped ends for connection to vehicle power;

A four-pin receptacle with cap for connection to joystick control;

An unused strain relief fitting for an additional operator station cable.

The valve shall have 3" NPT female inlet and outlet, stainless steel ball and housing, valve position indicator, and six-conductor cable with waterproof plug for connection to enclosure valve control cable. The valve kit shall be covered by a written five-year warranty.